1. OSI vs TCP/IP Layer

- OSI Model (7 layer):

  - Physical – kabel, sinyal. (alat: hub, repeater)

  - Data Link – MAC address, Ethernet, VLAN. (alat: switch).

  - Network – IP, routing. (alat: router).

  - Transport – TCP, UDP.

  - Session – komunikasi antar aplikasi.

  - Presentation – format data (enkripsi, kompresi).

  - Application – HTTP, FTP, DNS.

- TCP/IP Model (4 layer):

  - Network Access (Physical + Data Link)

  - Internet (Network)

  - Transport (Transport)

  - Application (Session + Presentation + Application)

Contoh Protokol:

- Physical: Ethernet

- Data Link: ARP

- Network: IP, ICMP

- Transport: TCP, UDP

- Application: HTTP, FTP, DNS

2. Encapsulation & Decapsulation

- Encapsulation: Data → Segment → Packet → Frame → Bits.

- Decapsulation: Kebalikannya saat data diterima.

- Header ditambahkan di setiap layer:

  - Transport: TCP/UDP header

  - Network: IP header

  - Data Link: Ethernet header + FCS

Ethernet & LAN

- Ethernet: Protokol standar untuk LAN (Layer 2).

- LAN: Jaringan lokal (kantor, rumah), berbasis Ethernet.

- Frame Ethernet:

  - Header: Destination MAC | Source MAC | Type

  - Data: Payload

  - FCS: Frame Check Sequence (cek error).

3. LAN, MAN, WAN + Topologi

- LAN: Jaringan lokal (kantor, rumah).

- MAN: Jaringan antar kota.

- WAN: Jaringan global (Internet).

- Topologi:

  - Star (paling umum), Bus, Ring, Mesh, Hybrid.

4. Fungsi Perangkat

- Switch: Layer 2, forward berdasarkan MAC.

- Router: Layer 3, forward berdasarkan IP.

- Firewall: Filter traffic (security).

5. TCP Handshake & TCP vs UDP

- TCP 3-way handshake:

  - SYN → 2. SYN-ACK → 3. ACK.

- TCP vs UDP:

  - TCP: reliable, connection-oriented (HTTP, FTP).

  - UDP: fast, connectionless (DNS, VoIP).

6. Huawei VRP & User Level

- Mode CLI:

  - User View → System View → Interface View.

- Command dasar: [Huawei] sysname R1 [Huawei] interface GigabitEthernet 0/0/1 [Huawei-GigabitEthernet0/0/1] ip address 192.168.1.1 24

- User Level:

  - 0 (monitor), 1 (user), 2 (operator), 3–15 (admin).

7. Subnetting & Broadcast Address

IP: 192.168.10.0/26

Step 1: Subnet mask

- /26 → 26 bit untuk network.

- 11111111.11111111.11111111.11000000 → /26

- Subnet mask = 255.255.255.192 (karena 192 = 11000000).

Step 2: Hitung jumlah host

- Host bits = 32 - 26 = 6.

- Jumlah host = 26-2=64-2=62.

Step 3: Hitung blok size

- Oktet terakhir mask = 192.

- Blok size = 256 - 192 = 64.

- Artinya setiap subnet punya 64 alamat IP.

Step 4: Tentukan range

- Subnet pertama: 192.168.10.0 – 192.168.10.63

  - Network address = 192.168.10.0

  - Broadcast address = 192.168.10.63

  - Host range = 192.168.10.1 – 192.168.10.62

- Subnet kedua: 192.168.10.64 – 192.168.10.127

  - Network = 192.168.10.64

  - Broadcast = 192.168.10.127

  - Host range = 192.168.10.65 – 192.168.10.126

Contoh :

0, 16, 32, 48, 64, 80, ...

- Subnet : 192.168.7.32/28

- 32-28 = 4

- 24 = 16

- Network = 192.168.7.32

- Broadcast = 32 + 16 - 1 = 47 → 192.168.7.47

- Host range = 192.168.7.33 - 192.168.7.46

- Jumlah host = 16 - 2 = 14 host

8. Static vs Dynamic Routing

- Static: manual, cocok untuk jaringan kecil.

- Dynamic: otomatis (OSPF, RIP).

- Command static: ip route-static 10.0.0.0 255.255.255.0 192.168.1.1

9. OSPF

- Packet Types: Hello, DD, LSR, LSU, LSAck.

- Adjacency Steps:

  - Init → 2-Way → Exstart → Exchange → Loading → Full.

- Konsep:

  - Area, Router ID, Cost, DR/BDR.

10. Route Selection

- Preference: nilai kecil lebih disukai.

- Cost/Metric: jalur dengan cost terendah.

- Longest Match: mask paling panjang menang.

11. CIDR & Route Summarization

- CIDR = Classless Inter-Domain Routing → pakai prefix (/22, /24).

- Summarization = gabungkan beberapa network jadi satu prefix.

Contoh Soal

Gabungkan:

- 192.168.0.0/24

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Langkah 1: Konversi ke biner

- 192.168.0.0 = 11000000.10101000.00000000.00000000

- 192.168.3.0 = 11000000.10101000.00000011.00000000

Langkah 2: Cari bit yang sama

- Sama sampai bit ke-22.

- Jadi prefix = /22.

Langkah 3: Summary route

- 192.168.0.0/22 → mencakup 192.168.0.0 – 192.168.3.255.

✅ Tips cepat:

- 4 subnet /24 → summary /22.

- 8 subnet /24 → summary /21.

Latihan Cepat

Gabungkan:

- 10.0.8.0/24

- 10.0.9.0/24

- 10.0.10.0/24

- 10.0.11.0/24 → Summary = 10.0.8.0/22.

12. Collision Domain vs Broadcast Domain

- Collision: satu segmen fisik (hilang dengan switch).

- Broadcast: satu VLAN (hilang dengan router).

13. Ethernet Frame & MAC

- Format: D.MAC | S.MAC | Type | Data | FCS.

- MAC: 48-bit, unik per NIC.

14. VLAN & Interface Types

- Tagging: 802.1Q (TPID 0x8100).

- Interface:

  - Access (1 VLAN), Trunk (multi VLAN), Hybrid (flexibel).

15. STP & RSTP

- STP:

  - Root bridge election (BID terkecil).

  - Port roles: Root, Designated, Alternate.

  - Port states: Blocking, Listening, Learning, Forwarding.

- RSTP:

  - Konvergensi cepat.

  - Port states: Discarding, Learning, Forwarding.

16. Inter-VLAN Routing

- Router-on-a-Stick: interface GigabitEthernet0/0/1.10 dot1q termination vid 10 ip address 192.168.10.1 24 arp broadcast enable

- Layer 3 Switch: interface vlanif 10 ip address 192.168.10.1 24

17. NAT vs NAPT

- NAT: ubah IP.

- NAPT: ubah IP + port (banyak private IP → satu public IP).

18. Jenis Address

- MAC Address: Alamat fisik (48 bit, tertanam di NIC).

- IP Address: Alamat logis (IPv4 32 bit, IPv6 128 bit).

- Unicast: 00 (bit pertama 0).

- Multicast: 01 (bit pertama 1).

- Broadcast: FF-FF-FF-FF-FF-FF.

19. IPv4 vs IPv6

- IPv4: 32 bit, format dotted decimal.

- IPv6: 128 bit, format heksadesimal, lebih banyak alamat.

20. VLAN

- Virtual LAN: Memisahkan broadcast domain.

- Tagging: 802.1Q (TPID 0x8100, VLAN ID 12 bit).

- Interface:

  - Access: 1 VLAN.

  - Trunk: Multi VLAN.

  - Hybrid: Bisa tagged/untagged.

21. STP (Spanning Tree Protocol)

- Fungsi: mencegah looping dengan memblok jalur redundant.

- Root Bridge: BID terkecil (Priority + MAC).

- Port Roles: Root, Designated, Alternate.

- Port States: Blocking, Listening, Learning, Forwarding.

- RSTP: Konvergensi cepat (Discarding, Learning, Forwarding).

22. Routing

- Direct: Otomatis untuk jaringan langsung.

- Static: Manual, cocok untuk jaringan kecil.

- Dynamic: OSPF, RIP.

- OSPF:

  - Packet types: Hello, DD, LSR, LSU, LSAck.

  - Adjacency steps: Init → 2-Way → Exstart → Exchange → Loading → Full.

23. Collision vs Broadcast Domain

- Collision domain: Tabrakan → solusi: switch.

- Broadcast domain: Semua host dalam VLAN → solusi: router/VLAN.

24. NIC, LLC, SNAP

- NIC: Network Interface Card (alamat MAC).

- LLC: Logical Link Control (kontrol komunikasi).

- SNAP: Subnetwork Access Protocol (identifikasi protokol).

- FCS: Frame Check Sequence (cek error).

25. Inter-VLAN Routing

- Router-on-a-Stick:

interface GigabitEthernet0/0/1.10

dot1q termination vid 10

ip address 192.168.10.1 24

arp broadcast enable

- Layer 3 Switch:

interface vlanif 10

ip address 192.168.10.1 24